
Maintenance Log
	Equipment
	Daily
	Weekly
	Every

Other

Week
	Monthly
	Every Other Month
	Every

Six

Months
	Yearly
	As Needed

	Sterilizer, Statim
	
	
	
	
	
	
	
	

	- Empty used water container
	
	
	
	
	
	
	
	XX

	- Lubricate cassette seal
	
	
	XX
	
	
	
	
	

	- Apply Stat Dry to cassette
	
	
	XX
	
	
	
	
	

	- Replace air filter
	
	
	
	
	
	XX
	
	

	
	
	
	
	
	
	
	
	

	Sterilizer, Pelton
	
	
	
	
	
	
	
	

	- Clean chamber
	
	
	
	
	XX
	
	
	

	- Spore test
	
	
	
	
	XX
	
	
	

	- Check door seals
	
	
	
	
	XX
	
	
	

	- Activate safety valve
	
	
	
	
	
	XX
	
	

	
	
	
	
	
	
	
	
	

	Chairs
	
	
	
	
	
	
	
	

	- Clean
	
	XX
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	Operatory light
	
	
	
	
	
	
	
	

	- Clean shield
	
	XX
	
	
	
	
	
	

	- Check for drift
	
	
	
	XX
	
	
	
	

	- Clean reflector
	
	
	
	
	
	
	
	XX

	
	
	
	
	
	
	
	
	

	Dental Unit
	
	
	
	
	
	
	
	

	- Clean out vacuum trap
	
	XX
	
	
	
	
	
	

	- Flush handpiece tubings
	XX
	
	
	
	
	
	
	

	- Clean vacuum lines
	
	
	XX
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	Nitrous Oxide System
	
	
	
	
	
	
	
	

	- Turn system on/off
	XX
	
	
	
	
	
	
	

	- Check rubber components for wear
	
	
	
	
	XX
	
	
	

	- Perform overnight pressure test
	
	
	
	
	
	XX
	
	

	
	
	
	
	
	
	
	
	

	Automatic Film Processor
	
	
	
	
	
	
	
	

	- Check chemicals
	XX
	
	
	
	
	
	
	

	- Clean racks
	
	XX
	
	
	
	
	
	

	- Change chemicals
	
	
	
	XX
	
	
	
	

	- Clean entire unit
	
	
	
	
	XX
	
	
	

	
	
	
	
	
	
	
	
	

	Compressor
	
	
	
	
	
	
	
	

	- Check oil level
	
	XX
	
	
	
	
	
	

	- Check moisture indicator
	
	
	XX
	
	
	
	
	

	- Check air intake filters
	
	
	
	XX
	
	
	
	

	- Change oil
	
	
	
	
	
	
	XX
	

	- Regenerate desiccant
	
	
	
	
	
	
	
	XX

	
	
	
	
	
	
	
	
	

	Vacuum Pump
	
	
	
	
	
	
	
	

	- Check and clean filter
	
	XX
	
	
	
	
	
	

	- Check for water leaks
	
	XX
	
	
	
	
	
	

	- Run cleaner through lines
	
	XX
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	X-ray Heads, Intraoral
	
	
	
	
	
	
	
	

	- Check arms for drift
	
	
	
	XX
	
	
	
	

	- Check tube head for oil leaks
	
	
	
	XX
	
	
	
	

	- Verify audible alert
	
	
	
	XX
	
	
	
	

	- Check lead aprons
	
	
	
	XX
	
	
	
	

	- Calibrate settings
	
	
	
	
	
	
	
	2 years

	
	
	
	
	
	
	
	
	

	X-ray, Panoramic
	
	
	
	
	
	
	
	

	- Verify audible alert
	
	
	
	XX
	
	
	
	

	- Check lead aprons
	
	
	
	XX
	
	
	
	

	- Check cassettes
	
	
	
	
	XX
	
	
	

	- Check cassette holder
	
	
	
	
	XX
	
	
	

	- Calibrate settings
	
	
	
	
	
	
	
	2 years

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	



